Using the Text

The text utilizes a variety of command boxes and descriptions to lead you through construction sequences. Also, see the Appendix for Pro/ENGINEER WILDFIRE 2.0 Quick Reference Cards. The following icons, symbols, short keys, and conventions will be used (command sequences are always in a box):

Click: Tools        Apply  OK   Revolve Tool  RMB  Define Internal Sketch  Sketch Plane--- Plane: select RIGHT datum  Reference: TOP [Fig. 5.7(a)]    MMB [Fig. 5.7(b)]  MMB (to accept References)   Toggle the grid on

Click: Tools  Options      Slide the bar down to the option or type. Option: default_dec_places  Value: 3  Enter  Slide the bar down to the option or type. Option: sketcher_dec_places  Value: 3  Enter  Option: sketcher_starts_in_2d  Value: no  Enter  Option: (type) def_layer  Value:   layer_axis datum_axes (Fig. 5.6)  Add/Change  Apply     Save a copy of the currently displayed configuration file  Name clamp.pro  Ok  Close   Create a new object        OK  Edit  Setup  Units    Close  MMB

Commands:

-  			Continue with command sequence or screen picks using LMB

- Create lines	icon (with description) indicates command to pick using LMB

Mouse or keyboard terms used in this text:

- LMB     		Left Mouse Button

or “Pick”		term used to direct an action (i.e., “Pick the surface”)

or “Click”		term used to direct an action (i.e., “Click on the icon”)

or “Select”		term used to direct an action (i.e., “Select the feature”)

- MMB     		Middle Mouse Button (accept the current selection or value)

or Enter		press Enter key to accept entry

or 		Click on this icon to accept entry

- RMB			Right Mouse Button

(toggles to next selection, or provides a list of available commands)

Shortcut Keys:

Ctrl+A (Activate) 			Ctrl+C (Copy)

Ctrl+D (Standard Orientation) 	Ctrl+F (Find)

Ctrl+G (Regenerate) 			Ctrl+P (Print)

Ctrl+R (Repaint) 			Ctrl+S (Save)

Ctrl+A (Activate) 			Ctrl+V (Paste)

Ctrl+Z (Undo) 			Ctrl+Alt+A (Select All)

(Refer to the Pro/ENGINEER Wildfire 2.0 Quick Reference Cards in the Appendix)