Hide Navigation << >> 

System Parameters for Drawings

The following table lists all system parameters available for use in drawings, classified according to functionality.

	PARAMETER NAME
	DEFINITION

	&d#
	Displays a dimension in a drawing note, where # is the dimension ID.

	&ad#
	Displays an associative dimension in a drawing note, where # is the dimension ID.

	&rd#
	Displays a reference dimension in a drawing note, where # is the dimension ID.

	&p#
	Displays an instance number of a pattern in a drawing note, where # is the pattern ID.

	&g#
	Displays a gtol in a drawing note, where # is the gtol ID.

	&<param_name>
	Displays a user-defined parameter value in a drawing note.

	&<param_name>:att_cmp
	An object parameter that indicates the parameters of the component to which a note is attached.

	&<param_name>:att_edge
	An object parameter that indicates the parameters of the edge to which a note is attached.

	&<param_name>:att_feat
	An object parameter that indicates the parameters of the feature to which a note is attached.

	&<param_name>:att_mdl
	An object parameter that indicates the parameters of the model to which a note is attached.

	&<param_name>:att_pipe_bend
	An object parameter that indicates the parameters of the pipe bend to which a note is attached.

	&<param_name>:att_spool
	An object parameter that indicates the parameters of the spool to which a note is attached.

	&<param_name>:EID_<edge_name>
	An object parameter that references edges.

	&<param_name>:FID_<feat_ID>
	An object parameter that includes a feature parameter in a note by ID.

	&<param_name>:FID_<FEAT_NAME>
	An object parameter that includes a feature parameter in a note by name. 

	&<param_name>:SID_<surface_name>
	An object parameter that references surfaces.

	&angular_tol_0_0
	Specifies the format of angular tolerance values in a note from one to six decimal places.

	&current_sheet
	Displays a drawing label indicating the current sheet number.

	&det_scale
	Displays a drawing label indicating the scale of a detailed view. You cannot use this parameter in a drawing note. Pro/ENGINEER creates this parameter with a view and places it in notes automatically. You can modify its value, but you cannot call it out in another note.

	&dtm_name
	Displays datum names in a drawing note, where name is the name of a datum plane. The datum name in the note is read-only, so you cannot modify it; unlike dimensions, a datum name does not disappear from the model view if included in a note. The system encloses its name in a rectangle, as if it were a set datum.

	&dwg_name
	Displays a drawing label indicating the name of the drawing.

	&format
	Displays a drawing label indicating the format size (for example, A1, A0, A, B, and so forth).

	&linear_tol_0_0
	Specifies the format of dimensional tolerance values in a note from one to six decimal places.

	&model_name
	Displays a drawing label indicating the name of the model used for the drawing.

	&parameter:d
	Adds drawing parameters to a drawing note, where parameter is the parameter name and :d refers to the drawing. .

	&pdmdb
	Displays the database of origin of the model.

	&pdmrev
	Displays the model revision.

	&pdmrev:d
	Displays the revision number of the model (where :d refers to the drawing).

	&pdmrl
	Displays the release level of the model.

	&scale
	Displays a drawing label indicating the scale of the drawing.

	&scale_of_view_detailed_bar
	 

	&sym(<symbolname>)
	Includes a drawing symbol in a note, where symbolname is the name of the symbol.

	&todays_date
	Displays a drawing label indicating the date on which the note was created in the form dd-mm-yy (for example, 2-Jan-92). You can edit it as any other nonparametric note, using Text Line or Full Note. 

 

If you include this symbol in a format table, the system evaluates it when it copies the format into the drawing. 

 

To specify the initial display of the date in a drawing, use the configuration file option "todays_date_note_format." 

	&total_sheets
	Displays a drawing label indicating the total number of sheets in the drawing.

	&type
	Displays a drawing label indicating the drawing model type (for example, part, assembly, etc.).

	&view_name
	Displays a drawing label indicating the name of the view. You cannot use this parameter in a drawing note. Pro/ENGINEER creates it with a view and places it in notes automatically. You can modify its value, but you cannot call it out in another note.

	&view_scale
	Displays a drawing label indicating the name of a general scaled view. You cannot use this parameter in a drawing note. Pro/ENGINEER creates it with a view and places it in notes automatically. You can modify its value, but you cannot call it out in another note.

	Pro/REPORT System Parameters
	 

	&asm.mbr.comp....
	Retrieves information about the component from the model data and displays it in the report table.

	&asm.mbr.cparam....
	Retrieves a given component parameter.

	&asm.mbr.cparams....
	Lists information pertaining to all component parameters for the current model.

	&asm.mbr.name
	Displays the name of an assembly member. To show tie wraps and markers, the region attributes must be set to Cable Info.

	&asm.mbr.param....
	Displays information about parameters in an assembly member. 

	&asm.mbr.type
	Displays the type (part or assembly) of an assembly member.

	&asm.mbr.User Defined
	Lists the specified user-defined parameter for the respective assembly components. Note that "&asm.mbr." can be used as a prefix before any 

user-defined parameter in an assembly member. 

	&dgm....
	 

	&fam....
	Retrieves family table information about the model.

	&harn....
	Shows cable harness parameters for 3-D harness parts and flat harness assemblies. 

	&lay....
	Retrieves layout information about the model.

	&mbr....
	Retrieves parameters about a single component. 

	&mdl....
	Retrieves information about a single model. 

	&prs....
	Retrieves process-specific report parameters used to create reports on the entire process sequence.

	&rpt....
	Displays information about each record in a repeat region.

	&weldasm....
	Retrieves welding information about the model.

	&asm.mbr.cblprm....
	Lists values for a given cabling parameters.

	&asm.mbr.cblprms....
	Lists values for cabling and wire parameters.

	&asm.mbr.connprm....
	Lists parameters for connector pins in flat harness assemblies.

	&asm.mbr.location...
	Lists the location callouts in a specified view or all views of the drawing in session.

	&asm.mbr.pipe....
	Shows pipeline, pipe segment, and Pro/REPORT bend information parameters.

	&asm.mbr.generic.name….
	Lists the generic name information for a family table instance in a table.

	&asm.mbr.topgeneric.name….
	Lists the top generic name information for a family table instance in a table when working with a nested family table.


[image: image1.png]



